| Code: 114388E Update: 05.04.2023 Worker safety instructions according to §14 GefStoffV Advice: The samples for risk assessments (Section 6 Hazardous Substance Ordinance/§ 6 GefahrstoffV0) and operating instructions (Section 14 Hazardous Substance Ordinance)/ § 14 GefahrstoffV0) have been created to the best of our knowledge and with the greatest possible care. Ecolab only creates operating instructions for its products on customer request. The operating instructions are based exclusively on the information in the safety data sheet of the product and Ecolab does not guarantee the correctness of the operating instructions for the specific application due to the lack of insight into the individual areas of application of the respective customer. In this respect, our samples can only serve as reference or example for your company. They do not discharge the employer from his responsibility referring to §§ 6 and 14 of the Hazardous Substances Ordinance/§§ 6 and 14 Gefahrstoff-Verordnung and must therefore be adapted by the customer to the local conditions and requirements related to the workplace obligatory. We ask for your understanding that Ecolab cannot be held responsible or accept any liability particularly with regard to the completeness, correctness and applicability of the samples provided. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Company: |  |  |  |  |  | Responsible physician or clinic: |  |  |
| Division: |  |  |  |  |  | Emergency telephone contact: |  |  |
| Position / Function: |  |  |  |  |  | First aid person: |  |  |
| Hazardous Materials Name |  |  |  |  |  |  |  |  |
| SEKUSEPT AKTIV UFI: C391-S8PQ-K10W-GW3P Instrument Disinfectant - For professional users only. Phrasecheck for Pure Substance or Substance Hazardous components which must be listed on the label: Sodium carbonate peroxide , Components: Oxygen-based bleaching agents, Phosphonates, Non-ionic surfactants, Perfumes, Disinfectants, The following information refer to the handling of undiluted product |  |  |  |  |  |  |  |  |
| Hazards Identification |  |  |  |  |  |  |  |  |
| Signal Word |  |  |  | Danger |  |  |  |  |
| Hazard Statements |  |  |  | H318 Causes serious eye damage. |  |  |  |  |
| Hazard class for water |  |  |  | WGK 1 |  |  |  |  |
| Protection measures and instructions |  |  |  |  |  |  |  |  |
|  |  |  | Do not get in eyes, on skin, or on clothing. Use only with adequate ventilation. Wash hands thoroughly after handling. Do not breathe dust. Do not mix with bleach or other chlorinated products – will cause chlorine gas. In case of mechanical malfunction, or if in contact with unknown dilution of product, wear full Personal Protective Equipment (PPE). |  |  |  |  |  |
|  |  |  | Eye/face protection: Safety goggles, Face-shield |  |  |  |  |  |
|  |  |  | Hand protection: No special protective equipment required. |  |  |  |  |  |
|  |  |  | Skin and body protection: No special protective equipment required. |  |  |  |  |  |
|  |  |  | Respiratory protection: When respiratory risks cannot be avoided or sufficiently limited by technical means of collective protection or by measures, methods or procedures of work organization, consider the use of certified respiratory protection equipment meeting EU requirements (89/656/EEC, (EU) 2016/425), or equivalent, with filter type: |  |  |  |  |  |
| Behaviour in danger situations |  |  |  |  |  |  |  |  |
|  |  | Suitable extinguishing media: Use extinguishing measures that are appropriate to local circumstances and the surrounding environment. Unsuitable extinguishing media: None known. Sweep up and shovel into suitable containers for disposal. |  |  |  |  |  |  |
| First aid |  |  |  |  |  |  |  |  |
|  |  | In case of eye contact: Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Get medical attention immediately. In case of skin contact: Rinse with plenty of water. If swallowed: Rinse mouth. Get medical attention if symptoms occur. If inhaledRemove to fresh air. Treat symptomatically. Get medical attention if symptoms occur. |  |  |  |  |  |  |
| Waste treatment |  |  |  |  |  |  |  |  |
|  |  | Dispose of in accordance with the European Directives on waste and hazardous waste. Waste codes should be assigned by the user, preferably in discussion with the waste disposal authorities. |  |  |  |  |  |  |
| Date: |  |  |  |  | Signing manager: |  |  |  |